Project 5 – Annotated Code Reference

Team: Jacob Kosmicki, Ryder Anderson, Harrison Drake

CIVE 202 – Spring 2026

| Syntax / Example | What It Does |
| --- | --- |
| Package Imports & Setup |  |
| import matplotlib.pyplot as plt | Loads the matplotlib plotting library for creating and customizing plots. |
| import numpy as np | Loads numpy for numerical operations like arrays and math functions. |
| import pandas as pd | Loads the pandas library for working with dataframes and tables. |
| from steelpy import aisc | Loads the steelpy AISC Shapes Database. This is a new package not used in previous projects. Contains all W-shape and WT-shape section properties from the AISC Steel Construction Manual v15.0. |
| !{sys.executable} -m pip install steelpy | Installs the steelpy package from PyPI if it is not already installed on the machine. |
| PHI_YIELDING_LRFD = 0.90 | Stores the LRFD resistance factor for tensile yielding as a constant so it can be reused across functions. |
| OMEGA_YIELDING_ASD = 1.67 | Stores the ASD safety factor for tensile yielding. |
| PHI_RUPTURE_LRFD = 0.75 | Stores the LRFD resistance factor for tensile rupture. |
| OMEGA_RUPTURE_ASD = 2.00 | Stores the ASD safety factor for tensile rupture. |
| SLENDERNESS_LIMIT = 300 | Stores the maximum recommended slenderness ratio L/r per AISC Spec Section D1. |
| Loading and Cleaning External Data (steelpy) |  |
| aisc.W_shapes.sections | Accesses the dictionary of all 289 W-shape sections stored in the steelpy database. |
| for name, section in w_sections.items(): | Loops through each W-shape section in the dictionary. name is the section designation, section is the object. |
| section.properties | Returns a dictionary of all section properties for one shape (area, bf, tf, d, ry, etc.). |
| props['name'] = name | Adds the section name to the properties dictionary so it becomes a column in the dataframe. |
| w_df = pd.DataFrame(rows) | Creates a pandas DataFrame from the list of property dictionaries. Each dictionary becomes one row. |
| w_df[['name', 'weight', 'area', ...]].copy() | Selects only the columns needed for tension member design from the full dataframe. .copy() prevents modifying the original. |
| w_clean.columns = ['Section', 'Ag (in^2)', ...] | Renames columns to be more readable with units included. |
| w_clean.replace('–', np.nan) | Replaces dash characters in the database with NaN so pandas treats them as missing values. |
| w_clean.isnull().sum() | Counts missing values in each column to verify the data is clean. |
| aisc.WT_shapes.sections | Accesses the dictionary of all 289 WT-shape sections from steelpy. |
| wt_df[['name', 'y']].copy() | Selects only the WT section name and centroid distance (y) which is needed for the shear lag factor. |
| Helper Functions for Section Property Lookup |  |
| def get_w_props(section_name): | Defines a function that takes a W-shape name as a string (e.g. 'W8X21') and returns a dictionary of its properties from steelpy. |
| aisc.W_shapes.sections[section_name] | Looks up one specific section by name from the steelpy database. |
| return props | Sends the properties dictionary back out of the function so it can be used to fill in design inputs. |
| def get_wt_xbar(wt_name): | Defines a function that takes a WT-shape name (e.g. 'WT4X10_5') and returns the centroid distance. |
| return section.properties['y'] | Returns only the centroid distance value from the WT-shape properties dictionary. |
| get_w_props('W8X21') | Calls the helper function to pull W8x21 properties. Returns area=6.16, bf=5.27, tf=0.400, d=8.28, ry=1.26. |
| get_wt_xbar('WT4X10_5') | Calls the helper function to get the WT4x10.5 centroid distance. Returns x_bar=0.831 in. |
| Tensile Yielding Function (AISC Spec Eq. D2-1) |  |
| def tensile_yielding(Fy, Ag): | Defines a function that checks tensile yielding on the gross section. Takes yield strength and gross area as inputs. |
| Pn = Fy * Ag | Calculates the nominal tensile yielding strength per AISC Specification Equation D2-1. |
| LRFD = PHI_YIELDING_LRFD * Pn | Multiplies nominal strength by phi_t = 0.90 to get the LRFD available strength. |
| ASD = Pn / OMEGA_YIELDING_ASD | Divides nominal strength by omega_t = 1.67 to get the ASD available strength. |
| return Pn, LRFD, ASD | Returns all three values so the wrapper function can use them. |
| Tensile Rupture Function (AISC Spec Eq. D2-2) |  |
| def tensile_rupture(Fu, Ag, num_holes, dh, tf, bf, d, conn_length, x_bar): | Defines a function that checks tensile rupture on the net section. Takes 9 inputs covering material, section, and connection properties. |
| An = Ag - num_holes * dh * tf | Calculates net area by subtracting bolt hole material from gross area per AISC Spec Section B4.3b. |
| U_min = (2 * bf * tf) / Ag | Calculates the minimum shear lag factor for open cross sections as the ratio of connected flange area to gross area. |
| U_case2 = 1 - (x_bar / conn_length) | Calculates U from Case 2 of AISC Spec Table D3.1. x_bar is the centroid distance of the WT cut. |
| if bf >= two_thirds_d: U_case7 = 0.90 | Case 7 from AISC Table D3.1. If flange width >= 2/3 of depth, U = 0.90 for 3 or more fasteners per line. |
| else: U_case7 = 0.85 | If flange width < 2/3 of depth, U = 0.85 for 3 or more fasteners per line. |
| U = max(U_min, U_case2, U_case7) | Uses the largest of the three U values per AISC specification. |
| Ae = An * U | Calculates effective net area per AISC Spec Equation D3-1. |
| Pn = Fu * Ae | Calculates nominal tensile rupture strength per AISC Spec Equation D2-2. |
| LRFD = PHI_RUPTURE_LRFD * Pn | Multiplies nominal strength by phi_t = 0.75 for LRFD available rupture strength. |
| ASD = Pn / OMEGA_RUPTURE_ASD | Divides nominal strength by omega_t = 2.00 for ASD available rupture strength. |
| return Pn, LRFD, ASD, An, Ae, U | Returns 6 values including intermediate results for reporting. |
| Slenderness Check Function (AISC Spec Section D1) |  |
| def slenderness_check(L_ft, ry): | Defines a function that checks the recommended slenderness limit. Takes member length in feet and radius of gyration. |
| L_in = L_ft * 12 | Converts the member length from feet to inches since ry is in inches. |
| ratio = L_in / ry | Calculates the slenderness ratio L/r. |
| passes = ratio <= SLENDERNESS_LIMIT | Returns True if L/r is at or below 300, False if it exceeds the limit. |
| Main Wrapper Function |  |
| def tension_member_design(name, Fy, Fu, Ag, bf, tf, d, ry, x_bar, num_holes, dh, conn_length, L_ft, DL, LL): | Defines the main wrapper function that runs all three checks. Takes 15 inputs covering the section name, material, geometry, connection, length, and loads. |
| Pu = 1.2 * DL + 1.6 * LL | Calculates the LRFD required strength using ASCE 7 load combination. |
| Pa = DL + LL | Calculates the ASD required strength. |
| Pn_y, LRFD_y, ASD_y = tensile_yielding(Fy, Ag) | Calls the yielding function and unpacks the 3 returned values into separate variables. |
| Pn_r, LRFD_r, ASD_r, An, Ae, U = tensile_rupture(...) | Calls the rupture function and unpacks all 6 returned values. |
| slender_ratio, slender_ok = slenderness_check(L_ft, ry) | Calls the slenderness function and unpacks the ratio and pass/fail result. |
| if LRFD_y <= LRFD_r: | Compares the two LRFD available strengths. The smaller one governs the design. |
| governing = 'Tensile Yielding' | Sets the governing limit state string based on which available strength is lower. |
| f'Pu = {Pu:.1f} kips' | An f-string that formats the number to 1 decimal place. The :.1f controls the formatting. |
| "o.k." if LRFD_y >= Pu else "N.G." | A conditional expression that prints o.k. if the available strength meets the required strength, N.G. if it does not. |
| results = { 'name': name, 'Pu': Pu, ... } | Stores all results in a dictionary so they can be used for plotting and summary tables later. |
| return results | Returns the dictionary so the plotting code can access the values. |
| Running Scenarios and Verification |  |
| s1_props = get_w_props('W8X21') | Pulls the W8x21 properties from steelpy into a dictionary for Scenario 1. |
| s1_xbar = get_wt_xbar('WT4X10_5') | Pulls the WT4x10.5 centroid distance for the shear lag calculation. |
| s1 = tension_member_design(name='Scenario 1: W8x21', ...) | Runs the full design check for Scenario 1 and stores the results dictionary in s1. |
| s2_props = get_w_props('W10X33') | Pulls W10x33 properties for Scenario 2. Larger section with more capacity. |
| s2_xbar = get_wt_xbar('WT5X16_5') | Pulls the WT5x16.5 centroid distance for Scenario 2. |
| s3_props = get_w_props('W6X15') | Pulls W6x15 properties for Scenario 3. Smaller section where rupture governs more heavily. |
| s3_xbar = get_wt_xbar('WT3X7_5') | Pulls the WT3x7.5 centroid distance for Scenario 3. |
| Summary Table |  |
| summary = pd.DataFrame([s1, s2, s3]) | Creates a pandas DataFrame from the three scenario result dictionaries. Each dictionary becomes one row. |
| summary = summary[['name', 'Pu', 'Pa', ...]] | Reorders the columns to put them in a logical reading order. |
| summary.columns = ['Scenario', 'Pu (kips)', ...] | Renames columns to include units for readability. |
| summary = summary.round(1) | Rounds all numeric values to 1 decimal place. |
| Plotting |  |
| scenarios = [s1, s2, s3] | Creates a list of all three result dictionaries for easy looping. |
| [s['LRFD_yielding'] for s in scenarios] | A list comprehension that extracts the LRFD yielding value from each scenario dictionary. |
| x = np.arange(3) | Creates an array [0, 1, 2] for the x-axis positions of the three scenario groups. |
| width = 0.25 | Sets the bar width so three bars fit side by side at each x position. |
| fig, ax = plt.subplots(figsize=(10, 6)) | Creates a figure and axes object. figsize sets width and height in inches. |
| ax.bar(x - width, lrfd_yield, width, label='Yielding', color='steelblue') | Plots the yielding bars shifted left by one bar width. label sets the legend text. |
| ax.bar(x, lrfd_rupt, width, label='Rupture', color='coral') | Plots the rupture bars at the center position. |
| ax.bar(x + width, required_Pu, width, label='Required', color='gray') | Plots the required strength bars shifted right by one bar width. |
| ax.set_xticks(x) | Sets the x-axis tick positions to the center of each group. |
| ax.set_xticklabels(['S1: W8x21', 'S2: W10x33', 'S3: W6x15']) | Labels each group of bars with the scenario section name. |
| ax.legend(fontsize=10) | Adds a legend showing which color corresponds to yielding, rupture, and required. |
| ax.grid(True, axis='y', alpha=0.3) | Adds horizontal gridlines at 30% opacity to make values easier to read. |
| plt.tight_layout() | Adjusts spacing so labels and titles are not cut off. |
| plt.show() | Renders and displays the completed plot. |